SECTION  09545—SPECIFICATION FOR AMERICAN DECORATIVE CEILINGS’ “CUSTOM OPEN BEAM 360” METAL CEILING SYSTEM

PART 1 – GENERAL

1.01  Related Work

A.  Insulation:  Section:__________

B.  Mechanical equipment:  Section __________

C.  Electrical:  Section __________

D.  Gypsum board systems:  Section __________

E.  Acoustical ceilings:  Section __________

1.02  System Description

A.  Provide the entire system as specified herein, and as detailed within the Contract Documents, and

installation drawings of the manufacturer.  The system shall be as described in Section 2.02 “Materials”.

1.03  Quality Assurance

A.  Subcontractor qualifications:  Installer shall have not less than three years of successful experience in the installation of Grid Cell metal ceiling systems on projects with requirements specified.

B.  Requirements of regulatory agencies:  Codes and regulations of authorities having jurisdiction.

C.  Source quality control:

1.  Test reports:  Manufacturer will provide test certification for minimum requirements as tested in

accordance with applicable industry standards and/or to meet performance standards specified by various agencies.

2.  Changes from system:  System performance following any substitution of materials or change in

assembly design must be certified by the manufacturer.

1.04  References.

A.  ASTM C636:  Manufacturing and Installation of Suspended Ceilings.

B.  ASTM E119:  Fire Tests of Building Construction and Materials.

C.  ASTM C423:  Sound Absorption and Sound Absorption Coefficients by the Reverberation Room

Method.

1.05  Submittals.

A.  Samples:  Submit representative sample of color and finish of all exposed materials.  Sample shall be approximately 11”x 11”.

B.  Installation Drawings:  Submit three copies each of typical installation details indicating system

components, splice details, and sequence of installation.

C.  Manufacturer’s data:  Submit manufacturer’s standard catalog cut sheet with sample above.

1.06  Delivery, Storage and Handling

A.  Delivery of materials:  Deliver materials in original, unopened packages clearly labeled with

manufacturer’s name and identification numbers.

B.  Storage:  Store in manner that will prevent warpage, scratches or damage of any kind.

C.  Handling:  Handle in such a manner as to insure against racking, distortion of physical damage of any kind.

1.07  Project Conditions

A.  Existing conditions:  (Include specific alteration work requirements for project, if any).

B.  Environmental requirements for interior installation:  Building shall be enclosed with windows and

exterior doors in place and glazed, and roof watertight before installation of metal ceiling system and related components.  Climatic condition range of 60 degrees Fahrenheit (15.56 degrees Celsius) to 85 degrees Fahrenheit (29.44 degrees Celsius) and relative humidity of not more than 70% before installation of metal ceiling components.

C.  Coordination with other work:

1.  Mechanical work:  Ductwork above ceiling shall be complete, and permanent heating and cooling

systems operating to climate conditions prior to installation of metal ceiling components.

2.  Electrical work:  Installation of conduit above ceiling shall be complete before installation of metal

ceiling components.

3.  Fire protection work:  Fire protection lines and/or equipment occurring above ceiling shall be

completed and tested before metal ceiling components are installed.

D.  Protection:  Protect completed work above ceiling system from damage during installation of metal

ceiling components.

PART 2 – PRODUCTS

2.01 Manufacturer

- Provide CUSTOM Open Beam 360 Metal Ceiling model number OB360/24XX-F7 as manufactured by American Decorative Ceilings, Inc.; 4510 E. 71st St.; Cleveland, Ohio  44105; Telephone number (216) 341-2222  (Contact:  Joseph Conte).

2.02 Materials

- System:  Provide CUSTOM Open Beam ceiling as per Architect’s sample.

- Main Beams in custom lengths (depending upon module size specified) shall be notched with center spacings to accommodate specified module size.

- Main Beam splice/ hanger shall be installed at the insertion of all abutting mains.  Additional Main Beam splice Hanger shall be installed along the main member at remaining slot locations at a spacing not to exceed 48” o.c.

- Cross Beams shall be in appropriate lengths and notched to accommodate specified module size as indicated on installation drawings.  To be installed perpendicular to Main Beams at all notched locations.

- Filler Beams shall be in module length and installed parallel to Main Beams and engaged into notches of Cross Beams.

- All Beam Members are to be Fully Enclosed, with no top opening.

- Wall Perimeter:  shall consist of Fully Enclosed P-molding with matching face and depth dimensions of Open Beam section.  P-molding to have a minimum top lip returned flange of ½” for added strength and stability.

- Finish to be color to match Architect’s sample (F47).

2.03 Fabrication

- Beams:  Fabricate and form beam members from appropriate thickness of aluminum to provide positive locking at all intersections without the use of additional fasteners or clips.  Factory finished to match sample in Architect’s possession.

- Accessory Items: Independently suspend all mechanical air distribution devices and all

Electrical lighting fixtures.

PART 3 – EXECUTION

3.01  Inspection

A.  Examine areas to receive metal ceiling system for conditions which will adversely affect installation.

B.  Do not start work until unsatisfactory conditions are corrected.

C.  Work to be concealed:  Verify work above support system is complete, tested, and installed in manner which will not affect layout and installation of metal ceiling system.

3.02  Preparation

A.  Field dimensions:  Installer must verify actual field dimension prior to installation.

B.  Coordination:  Coordinate and schedule installation of metal ceiling system with work of trades affected by this installation, with particular attention given to mechanical and electrical work required to be

installed and operating before ceiling work can begin.

A.  Provide the entire system as specified herein, and as detailed within the Contract Documents, and

installation drawings of the manufacturer.  The system shall be as described in Section 2.02 “Materials”.

3.03  Installation

A.  Reference:  Install in accordance with approved installation drawings and Contract Documents.

3.04  Adjust and Clean

A.  Adjust components for uniform tolerances.  Replace any and all ceiling panels that are scratched or

dented.

B.  Clean exposed surfaces with non-solvent based, non-abrasive commercial cleaner.

3.05  Extra Stock

A.  Provide (2-5% (depending on project size)) extra members in matching finish to owner’s                           representative.  Stock material to be labeled for storage.